NATIONAL SENIOR CERTIFICATE

GRADE 12

MECHANICAL TECHNOLOGY FEBRUARY/MARCH 2016

MEMORANDUM

MARKS:  200

This memorandum consists of 18 pages.

QUESTION 1:  MULTIPLE-CHOICE QUESTIONS

1.1         C                                                                                                                                           (1)

1.2         B                                                                                                                                            (1)

1.3         D                                                                                                                                           (1)

1.4         D                                                                                                                                           (1)

1.5         A                                                                                                                                            (1)

1.6         C                                                                                                                                           (1)

1.7         D                                                                                                                                           (1)

1.8         D                                                                                                                                           (1)

1.9         B                                                                                                                                            (1)

1.10       D                                                                                                                                           (1)

1.11       D                                                                                                                                           (1)

1.12       C                                                                                                                                           (1)

1.13       B                                                                                                                                            (1)

1.14       B                                                                                                                                            (1)

1.15       A                                                                                                                                            (1)

1.16       C                                                                                                                                           (1)

1.17       A                                                                                                                                            (1)

1.18       C                                                                                                                                           (1)

1.19       D                                                                                                                                           (1)

1.20       B                                                                                                                                            (1)

[20]

QUESTION 2:  SAFETY

2.1         Safety – Centre lathe

•    Look out for revolving work pieces 

•    Do not remove shavings by hand 

•    Be careful not to run the cutting tool into the chuck 

•	Do not make any adjustments on the work piece while the machine is running 

•    Do not leave tools on the machine while in operation 

(Any 2 x 1)       (2)

2.2         Safety – Tensile Tester

•    Use safety goggles 

•    Do not apply excessive pressure 

•    Test piece to be well secured for testing 

•    Check hydraulic fluid level 

(Any 2 x 1)       (2)

2.3         Safety – Spring Tester

•   Spring tester should be in a good condition 

•   Spring tester must be fitted correctly and firmly 

•   Ensure that the spring cannot slip out of position before applying the load



•   An uniform load must be applied 

•   Release the load carefully and uniformly 

(Any 2 x 1)       (2)

2.4         Safety – Cylinder Leakage

•    Clean the area around the spark plug, before removing the spark plug 

      To prevent dirt from falling into the cylinder.  

•    Be careful when removing radiator cap 

      The water may be hot and under pressure. 

•    Do not exceed the specified pressure to test the cylinder 

      To prevent damage to the seals and tester. 

•	The tester must fit properly and be well tightened in the spark hole or injector hole 

      To prevent damage to the tester and spark hole or injector hole. 

Any 2 x 2       (4)

[10]

QUESTION 3:  TOOLS AND EQUIPMENT

3.1         Compression testing

3.1.1         •   Wet compression test 

•   Dry compression test                                                                                     (2)

3.1.2         •    Worn cylinders 

•    Worn piston rings 

•    Worn piston 

•    Leaking inlet valve 

•    Leaking exhaust valve 

•    Leaking cylinder head gasket 

(Any 2 x 1)      (2)

| 3.2 | Oil pump • Oil pressure meter or oil pressure tester  | (1) |
| --- | --- | --- |
| 3.3 | Cooling system test • Remove the radiator cap and fit the tester  • Pump air at the prescribed pressure into the system  • Note the reading  and if the reading drops, it indicates a leaking |  |

system 

•    To test for a leaking cylinder-head gasket,  the engine is started. 

•	If the reading increases while the engine idles, it indicates on a leaking cylinder-head gasket                                                                                                  (7)

[12]

QUESTION 4:  MATERIALS

4.1         Iron-carbon properties

4.1.1         Pearlite:

•    Good ductility 

•    Hard 

•    Strong and tough 

•    Resistant to deformation 

(Any 2 x 1)      (2)

4.1.2         Cementite:

•    Intensely hard 

•    Brittle.                                                                                                        (2)

4.2         Critical points

4.2.1         AC1 – lower critical point

•	The lowest temperature to which steel must be heated to be hardened. 

•	The   lowest   temperature   where   the   structure   starts   to change. 

(Any 1 x 2)      (2)

4.2.2         AC3 – high critical point

•	The highest temperature to which the steel can be heated to obtain maximum hardness. 

•	The temperature where the steel completely loses its magnetic properties.  

•    The temperature where the steel's structure is at its finest. 

(Any 1 x 2)      (2)

4.3         Carbon content determines the hardness of steel.                                                       (1)

4.4         Cementite structure in steel determines the hardness.                                               (1)

4.5         Ferrite structure in steel determines the ductility.                                                         (1)

4.6         Austenite is a solid solution of iron and carbon also called iron carbide. 

The structure is at its finest.                                                                                             (2)

[13]

QUESTION 5: TERMINOLOGY

5.1         Gear calculation

5.1.1

5.1.2

Circular pitch = m× π

module = circular pitch                              

π

= 12,567                                       

π

= 4 mm                                         

Outside diameter = PCD + 2m



PCD = OD − 2m                               

=112 − 2(4)

=104 mm                               

(3)

(3)

5.1.3

CuttingDepth = 2,157m

CuttingDepth = 2,157 × 4

CuttingDepth = 2,25m          

OR       CuttingDepth = 2,25 × 4

= 8,628mm

= 9 mm           

= 8,63 mm

(2)

5.1.4

5.1.5

5.1.6

5.1.7

Addendum = m

= 4 mm

Dedendum =1,157m

=1,157 × 4

= 4,628mm

= 4,63 mm

Clearance = 0,157m

= 0,157 × 4

= 0,628 mm

= 0,63 mm

module =  PCD  Teeth

Teeth = PCD

m

= 104

4

= 26 teeth



Dedendum =1,25m            

OR      Dedundum =1,25 × 4

= 5 mm             

Clearance = 0,25m            

OR                           = 0,25 × 4

=1 mm             







(1)

(2)

(2)

(3)

5.2         Indexing

Indexing = 40 n

= 40

26

=  40 ÷ 2

26    2                                                                                    

= 20

13

=1 7  × 3

13   3

=1 21                                                                                         

39

Indexing =1 full turn of the crank and 21 holes on the 39 hole circle         



(4)

| 5.3 | Screw thread cutting • Set up the work piece in the lathe and turn the part to be threaded to the major diameter of the thread.  • Set the compound slide to the correct angle (30°) to the right and set the tool up accurately in the post.  • Consult the index plate of the quick-change gearbox for 2 mm pitch and move the levers accordingly.  • Start the lathe and set the cutting tool so that it just touches the work piece. Set graduated dials to zero (cross feed and compound slide)  • Move cutting tool a short distance off end of work piece and feed compound slide say 0,06 mm inwards.  • With lathe turning, engage half nuts at the correct line on the chasing dial, putting the first cut in progress.  • Withdraw the cutting tool quickly and disengage the half-nut lever. Return the carriage to the starting point of the thread.  OR Stop the machine, leave half nut engaged, back off slide past zero and return carriage to start position in reverse  |  |
| --- | --- | --- |
|  | • Check with thread gauge to see if thread pitch is correct.  • Repeat with successive cuts until thread is complete. (Remember to bring cross-feed collar back to zero for each cut)  • Each successive cut is set by means of the compound slide. Check thread with ring gauge for correct fit.  | (10) [30] |

QUESTION 6: JOINING METHODS

6.1         MIG/MAGS welding equipment

| 6.1.1 | MIG/MAGS welding equipment  | (1) |
| --- | --- | --- |
| 6.1.2 | Labels A = Shielding gas cylinder  B = Regulator  C = Gas flow meter  D = Continuous wire reel  E = Welding gun  F = Arc  G = Earth clamp  | (7) |
| 6.1.3 | Purpose Prevents oxygen  to come in contact with the molten metal. | (2) |

6.2         Weld defects

6.2.1         Defect: Slag inclusion

Causes:

•    Included angle is too narrow. 

•    Rapid chilling. 

•    Weld temperature is too low. 

•    High viscosity of molten metal. 

•    Slag from previous run weld not removed. 

(Any 2 x 1)      (2)

6.2.2         Defect: Undercutting

Causes:

•    Faulty electrode manipulation. 

•    Current too high. 

•    Arc length too long. 

•    Speed of weld too fast. 

(Any 2 x 1)      (2)

6.3         Welding defects

6.3.1         Defect: Lack of fusion

Preventions:

•	Adjust   the   electrode   angle   and   prepare   the   V   groove properly.

•    Weave must be sufficient to melt sides of the joint. 

•    Proper current will allow fusion. 

•    Adjust welding speed to ensure fusion. 

(Any 2 x 1)      (2)

6.3.2         Defect: Weld craters

Preventions:

•    Use lower current. 

•    Use proper welding technique. 

•    Use correct electrode 

(Any 2 x 1)      (2)

6.4         Dye penetration test

•    Clean the weld that needs to be tested. 

•    The dye is sprayed onto the clean surface. 

•    Allow the dye to penetrate the weld joint. 

•    Excess dye is cleaned away with a cleaning agent. 

•    Allow surface to dry thoroughly. 

•	Spray a developer onto the surface to bring out the dye trapped in the crack. 

•    The dye will show all the surface defects                                                               (7)

[25]

QUESTION 7:  FORCES

7.1         Resultant

1,5sin40°

1,5 kN

1,5cos40°

2,1 kN

40°

50°

90°

4,7kN

50°

130°

3,1sin50°

3,1 kN

3,1cos50°

7.1.1 ∑ HC = 4,7 - 3,1cos50° - 1,5cos40°

= 4,7 - 1,99 - 1,15

= 1,56k N

7.1.2 ∑ VC = 2,1 + 1,5sin40° − 3,1sin50°

= 2,1 + 0,96 − 2,37

= 0,69 kN









| 7.1.3 Horizontal components | Magnitudes | 7.1.4 Vertical components | Magnitudes |
| --- | --- | --- | --- |
| 4,7 kN | 4,7 kN  | 2,1 kN | 2,1 kN  |
| 3,1 kN Cos50° | -1,99 kN  | 1,5 kN Sin40 0 | 0,96 kN  |
| 1,5 kN Cos40° | -1,15 kN  | 3,1 kN Sin50° | -2,37 kN  |
| TOTAL | 1,56 kN  | TOTAL | 0,69 kN |

E2  = HC 2  + VC 2

E  =   1,56 2  + 0,69 2

                       1,56kN     

0,69kN

E  = 1,71 kN                                                

R

θ

Tan

=  VC HC

= 0,69

1,56

θ

= 23,86 0

23,86°





E   = 1,71k N

at 23,86 0 north from east       

(15)

7.2         Stress and Strain

Forces

Force = load× gravity

= 600 ×10

π

2

Area = D

π

4



× 0,016 2

=

= 6000N     

4

= 2,011×10 − 4 m2    

Stress = Force                        

Area

=      6000

2,011×10 −4                          

= 29841551, 83 Pa

= 29,84 MPa                

(6)

7.3         One Pascal (1 Pa) is equal to one Newton force (1 N)  acting onto  an

area of one square metre (1 m2)                                                                                      (3)

7.4         Reactions

Taking moments around A

=

(B × 3,5) + (1400 × 0,7) = (350 × 6,7)2,65 + (1600 × 6)              

3,5B + 980 =6214,25 + 9600                              

3,5B =  6214,25 + 9600 − 980

3,5

B = 4238,36

3,5                                  

N

Taking moments around B

=

(A × 3,5) + (1600 × 2,5) = (350 × 6,7)0,85 + (1400 × 4,2)     

3,5A + 4000 =1993,25 + 5880                         

3,5A = 1993,25 + 5880 - 4000

3,5

A =1106,64

3,5

N



(6)

[30]

QUESTION 8: MAINTENANCE

8.1         Routine maintenance.

•    Tear on the belt.

•    Misalignment of belt drive. 

•    Overheating of components. 

•    Belt slip. 

•    Belt wear. 

•    Pulley wear.  

•    Financial loss due to the damage suffered.

•    Loss of valuable production time. 

(Any 2 x 1)       (2)

8.2         Cutting fluid

•    To allow it to flow easily 

•    Dissipate excess heat 

•    Prevent excessive load on pump 

(Any 2 x 1)       (2)

| 8.3 | Flash point Is the lowest temperature at which ignite.  | the | oil | gives | off | a | vapour | which | will | (2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8.4 | 'API' American Petroleum Institute  |  |  |  |  |  |  |  |  | (2) |

8.5         Automatic transmission fluid

•    Transmit power in the torque convertor 

•	Let hydraulic fluid transmit energy in order to move various parts such as the servo unit. 

•	Acts as heat transfer medium to transfer heat within the transmission to outside and assist in cooling it down. 

•    Acts as a lubricant for gears and bearings. 

(Any 2 x 1)             (2)

8.6         Replace belt in a drill press

•    Machine should be switched off the locked out. 

•	Tension on the belt to be released by loosening an adjusting screw or releasing the belt tensioner. 

•    Remove the belt. 

•    Replace with new belt of the correct type and size. 

•    The belt should be re-tensioned and aligned. 

(5)

[15]

QUESTION 9:  SYSTEMS AND CONTROL

9.1         Gear drive

9.1.1         Rotational frequency of the electric motor:

NA  =  TB × TD                                                                            

ND        TA × TC

N  =  80 × 63 × 2

A            30 × 40                                           

10080

NA =



1200

NA = 8,4 r/s                                                 

(5)

9.1.2         Speed ratio of gear train:

Speed ratio =  Input   Output

Speed ratio = Driven teeth

Driver teeth

=  8,4

          OR

=  80 × 63              

2                                                                     30    40              

= 4,2 : 1            

= 4,2 : 1

(2)

9.2         Belt drive

9.2.1         Diameter of the driven pulley

N1 × D1 = N2 × D2

1

N  = N2  × D2

D1                                                               

=  7,2 × 600                              

800

= 5,4 r/s                                 

(3)

9.2.2         Power transmitted:

π

P = (T1 − T2 )

π

Dn

P =(  300 −120)

× 0,6 × 7,2               

T1  = 2,5

T2

2

T = 300            

= 2442,90 Watt

= 2,44 kW

2,5

                             = 120 N

OR

π

P = (T1 − T2 )

π

Dn

P =(  300 −120)

× 0,8 × 5,4        

T1  = 2,5

T2

2

T = 300

= 2442,90 Watt

= 2,44 kW

                                        2,5             

= 120 N

(3)

9.3         The volume of gas can be changed by the altering of ...

•   its pressure 

•   its temperature 

•   both its pressure and temperature 

(Any 2 x 1)        (2)

9.4         Definition of Boyle's law

The  volume  of  a  given  mass  of  gas  is  inversely  proportional  to  the pressure on it, if the temperature remains constant                                                  (3)

9.5         Hydraulics

9.5.1         Fluid pressure

π

2

A A  = D

π

4

2

= 0,04

4

= 1,26 × 10 -3 m2                                                         

A

P	=  F   A A

=      80      Pa                                 

1,26 ×10 -3

= 63661,98 Pa

= 63,66 kPa                                       

(3)

9.5.2         Diameter of piston B

PB  = PA

B

P   =  FB

A B                                                                                      

B

A   = FB

PB

A   =      320

B       63492,06

B

A   = 5,04 × 10 -3                                                                   

π

2

A = D

4                                                  

π

DB =

AB × 4

π

5,04 × 10 −3 × 4

=

= 0,08 m

= 80 mm                                           

(4)

[25]

QUESTION 10: TURBINES

10.1       Water turbine blades

To supply water pressure  to the turbine                                                                           (2)

10.2       Reverse flow

•   Deriaz 

•   Francis                                                                                                                                             (2)

10.3       Supercharger

•    Roots

•    Twin screw

•    Centrifugal 

•    Vane 

(Any 2 x 1)       (2)

| 10.4 | Turbocharger • Exhaust gases drive the turbine  • The turbine drives a compressor via a common shaft • The compressor forces  compressed air above atmospheric pressure | (5) |
| --- | --- | --- |
|  | • Exhaust gases leave system through the exhaust pipe  |  |
| 10.5 | Supercharger over turbocharger |  |
|  | • Do not suffer lag  |  |
|  | • More efficient at low revolution per minute.  |  |
|  | • Does not require extensive exhaust modification. |  |
|  | • No special shutdown procedure is required.  (Any 2 x 1) | (2) |
| 10.6 | Lag |  |
|  | Lag is the delay  between pressing the accelerator pedal  and feeling the pressure building up. | (3) |
| 10.7 | Supercharger drive |  |
|  | • Belt drive  |  |
|  | • Gear drive  |  |
|  | • Chain drive  (Any 2 x 1) | (2) |

into the cylinder 

10.8       Gas turbine disadvantages

•	Cost is much greater than for a similar-sized reciprocating engine since the materials must be stronger and more heat resistant. 

•    Manufacturing operations are also more complex. 

•	Usually  less  efficient  than  reciprocating  engines,  especially  at  idling speed. 

•    Delayed response to changes in power settings. 

(Any 2 x 1)       (2)

[20]

TOTAL:       200