NATIONAL SENIOR CERTIFICATE

GRADE 12

MECHANICAL TECHNOLOGY NOVEMBER 2015

MEMORANDUM

MARKS:  200

This memorandum consists of 19 pages.

QUESTION 1:  MULTIPLE-CHOICE QUESTIONS

| 1.1 | B  | (1) |
| --- | --- | --- |
| 1.2 | B  | (1) |
| 1.3 | B  | (1) |
| 1.4 | A  | (1) |
| 1.5 | C / D | (1) |
| 1.6 | B  | (1) |
| 1.7 | A  | (1) |
| 1.8 | D  | (1) |
| 1.9 | A  | (1) |
| 1.10 | D  | (1) |
| 1.11 | D  | (1) |
| 1.12 | C  | (1) |
| 1.13 | B  | (1) |
| 1.14 | B  | (1) |
| 1.15 | A  | (1) |
| 1.16 | B  | (1) |
| 1.17 | A  | (1) |
| 1.18 | D  | (1) |
| 1.19 | A  B for Afrikaans only | (1) |

| 1.20 | C  | (1) |
| --- | --- | --- |
|  |  | [20] |

Copyright reserved                                                                                                           Please turn over

QUESTION 2:  SAFETY

2.1         Personal Safety Angle grinder

•       Wear eye safety protection

•       Wear ear plugs or muffs

•       Wear safety boots with steel toe caps

•       Wear overalls Leather apron

•       Wear gloves

(Any 3 x 1)           (3)

2.2         Safety – Hydraulic Press

•	The  predetermined  pressure  of  the  hydraulic  press  must  not  be exceeded

•       Ensure the pressure gauges is in a working order  

•	Platform on which the work piece rests must be rigid and square with the cylinder of the press 

•       The prescribed equipment must be used

•       Check that securing pins for the platform are fitted properly

•       Check on hydraulic pipes for leaks/ oil on floor

•       Bearing need to be placed in a suitable jig

(Any 3 x 1)           (3)

2.3         Safety – Spring tester

•	Be very careful that the jaws/clamp of the spring tester does not slip out

•	Use correct attachments of the valve spring tester to compress the spring. 

•	Do not stretch or compress the spring more than indicated in the specification 

(Any 2 x 1)           (2)

| 2.4 | Safety – Bearing and Gear puller • Make sure that the puller is the right one to use  • Do not use a hammer on the puller  • Use the correct spanner to tighten the clamps and to pull off the object  • Make certain that the puller is at a 90° to the work piece  • Legs of the puller must not be worn • Use the slip cover to prevent injury • When working with the puller do not work directly behind the puller in case it slips (Any 2 x 1) | (2) |
| --- | --- | --- |
|  |  | [10] |

QUESTION 3:  TOOLS AND EQUIPMENT

3.1          Tests

3.1.1      A cylinder leakage tester is used to check whether gases leak

from the cylinder in the engine during compression stroke.                        (2)

3.1.2      The  purpose  of  the  fuel  pressure  tester  is  to  test  the  fuel operating pressure in the system  and fuel pressure in the fuel

line that runs to the direct injection.                                                                 (2)

3.1.3      The purpose of the torsion tester is to investigate the relationship between momentum or torque applied to material and influence of

material length and torsional deflection.                                                      (2)

3.2         Reasons to perform cylinder leakage test

•       Power loss

•       Low compression

•       To determine whether cylinder head gasket has blown

•       Oil consumption due to excessive leakage past the piston rings

•       To identify leaking valves as a cause of excessive smoking

(Any 2 x 1)           (2)

3.3         Reasons for high CO reading

•       Rich mixture setting

•       Incorrect idle speed

•       Clogged air filter

•       Faulty choke

•       Faulty injectors

(Any 2 x 1)           (2)

| 3.4 | Tests that can be performed using a multi-meter • Current flow • Voltage test Battery • Resistance test • Transistor test • Continuity test • Temperature • Diode and capacitor testing (Any 2 x 1) | (2) |
| --- | --- | --- |
|  |  | [12] |

4.1         Iron-carbon equilibrium diagram

4.2         Iron-carbon Structures

(9)

4.2.1         Pearlite is the combination of ferrite and cementite   and it

contains 0,83% of carbon content before heat treatment                        (2)

| 4.2.2 | Cementite is formed when carbon content rises above 0,83%,  the carbon combines with pearlite crystals to form a very hard structure.  | (2) |
| --- | --- | --- |
|  |  | [13] |

QUESTION 5:  TERMINOLOGY

5.1         Calculation – spur gear

5.1.1

Module =

PCD T

=  108             

36

= 3                 

(2)

5.1.2         OD = PCD + 2m

= 108 + 2(3)       

= 108 + 6            

= 114 mm                                                                                                 (3)

5.1.3         Cutting depth  = 2,157 m              or                  2,25 m

= 2,157 x 3                                 2,25 x 3  

= 6,47 mm                                  6,75 mm  

(2)

5.1.4         Addendum    = m

= 3 mm   

(1)

5.1.5         Dedendum   = 1,157 m                or                  1,25 m

= 1,157 x 3                                    1,25 x 3   

= 3,47 mm                                     3,75 mm                       (2)

5.1.6         Circular pitch = m x 𝜋

= 3 x 𝜋                         

= 9,43 mm             or 9,42 mm 

(2)

5.1.7         Clearance     = 0,157 m                or                  0,25m

= 0,157 x 3                                    0,25 x 3   

= 0,47 mm                                     0,75 mm      

5.2         Advantages for compound slide:

•	The chips have a better chance of curling away, which prevents tearing of the thread. This results in a better finish. 

•	The left edge of the cutting tool performs most of the work whilst the right edge helps to polish the thread. 

•	The load on the tip of the cutting tool is less than the cross-slide method. 

•   If the cutting tool has broken down, it is easy to pick up the thread 

•   Faster than the cross slide method 

(2)

•   Can cut a larger screw pitch                                                 (Any 2 x 1)          (2)

5.3         Disadvantages screw cutting – cross-slide method:

•	The point of the tool, which is the weakest part of the tool, does most of the cutting. 

•	Because both edges of the tool do the cutting, two chips curl onto each other. This can result in a torn thread. 

•   A large load can damage the cutting tool/cutting edge. 

•   Slower method 

(Any 2 x 1)          (2)

5.4         Indexing:

Indexing =  40 n

=  40

72

=  10 × 3



OR   5 × 6                              

18   3             9   6

=  30

54                                                        

No full turns and 30 holes in a 54 - hole plate        

(4)

5.5         Advantages of Up-cut milling

•   A quick feed may be used   

•   Vibration experienced is less   

•   Less strain on the cutter and arbor  

•	There is a positive pressure on the feed screw spindle and nuts because the direction of the cutter is against the direction of the feed   

•	Metals with hard scale, start the cut under the scale where the metal is softer, this extends the life of the cutter   

•   More accurate (precise)   

•   Better finish   

(Any 2 x 1)          (2)

5.6        Disadvantages of Down-cut milling

•   A fine feed must be used   

•   Vibration of the arbor is unavoidable   

•	The cutter will come into contact with the hard scale of  a scale material, which is harmful to the cutter teeth

•   Cutter get blunt more easily   

•   Poor finish   

•   Slack on the table-feed must be eliminated   

(Any 2 x 1)          (2)

5.7         Calculate: parallel key

5.7.1

Width = D

4



=  42

4                           

= 10,5 mm

(2)

5.7.2

Thickness = D

6

=  42                  

6

= 7 mm              

(2)

[30]

QUESTION 6:  JOINING METHODS

6.1         Shielding gas

It forms the arc plasma, stabilises the arc on the metal being welded, and shields the arc and molten weld pool.

•  Reduces atmospheric contamination

•  It reduces excessive spatter and sparks                                     any 1 x 2             (2)

| 6.2 | Relationship between voltage and wire feed |  |
| --- | --- | --- |
|  | Higher voltage results in a higher melt rate therefore you need a higher |  |
| 6.3 | feed rate. Weld defects (causes) | (3) |

6.3.1         Slag inclusion

•   Included angle too narrow

•   Rapid chilling

•   Welding temperature to low / current to low

•   High viscosity of molten metal

•   Slag not removed from previous weld run

•   Current setting to low

•   Correct welding technique

•   Surface contamination                                        (Any 2 x 1)              (2)

| 6.3.2 | Incomplete penetration |  |  |
| --- | --- | --- | --- |
|  | • Speed too fast  |  |  |
|  | • Joint design faulty |  |  |
|  | • Electrode too large |  |  |
|  | • Current too low |  |  |
|  | • Wrong welding technique | (Any 2 x 1) | (2) |

6.4        Weld defects (preventative)

| 6.4.1 | Porosity • Use correct current • Hold a longer arc • Use correct electrodes • Check for impurities • Must shield the weld • Correct welding technique (Any 2 x 1) | (2) |
| --- | --- | --- |
| 6.4.2 | Lack of fusion • Use correct welding technique • Use the correct size of electrode • Use the correct current setting • Prepare the plate bevel/V-groove accordingly • Correct welding technique (Any 2 x 1) | (2) |

| 6.7 | Transceiver A unit that is used to send a sound wave (transmit)  and then act as a receiver to listen to the ultrasonic wave as it reflected through the metal.  • To determine defects  | (2) |
| --- | --- | --- |
|  |  | [25] |

6.5         Destructive tests

6.5.1         Free bend test

•	Measures the ductility of the weld deposit and the heat- affected area adjacent to the weld. 

•   To determine the percentage of elongation of the weld.                  (2)

6.5.2         Nick break test

•   It determines the internal quality of the weld  and can

reveal an internal defect if present.                                                     (2)

6.5.3         Machinability test

•	It  is  used  to  determine  the  weld's  hardness  and  its strength. 

•   To determine the machinability of the weld                                     (2)

6.6         Atmospheric contamination (MIGS/MAGS welding)

•   Inadequate shielding gas-flow

•	Excessive shielding gas flow (this can cause aspiration of air into the gas stream)

•   A severely blocked gas nozzle or a damaged gas supply system

(leaking hoses, fittings etc.) 

•   Excessive wind in the welding area (this can blow away the gas

shield)                                                                                                                          (4)

QUESTION 7:  FORCES

7.1         Equilibrant

280sin50°

280cos50°

280 N

200sin35°

300sin30°

300 N

300cos30°

30°

35°

50°

350 N

200 N

200cos35°

7.1.1

7.1.2

∑ HC = 280cos50° - 200cos35 - 300cos30° − 350

= 179,98 - 163,83 - 259,81 − 350

= - 593,66N

∑ VC = 280sin50° + 300sin30° − 200sin35°

= 214,49 + 150,0 − 114,72

= 249,77 N

 



 



(5)

(4)

OR

| 7.1.1 Horizontal components | Magnitudes | 7.1.2 Vertical components | Magnitudes |
| --- | --- | --- | --- |
| 300NCos30 0 | -259,81 N  | 280NSin50 0 | 214,49N  |
| 200NCos35 0 | -163,83 N  | 300NSin30 0 | 150,0 N  |
| 350 N | -350 N  | 0 N | 0 N |
| 280NCos50 0 | 179,98 N  | 200Sin35 0 | -114,72N  |
| TOTAL | -593,66 N  | TOTAL | 249,77 N |

7.1.3

E2  = HC2 + VC2                                                          

E  =   − 593,66 2 + 249,77 2                                

E  = 644,06 N

249,77N

                     22,82°

E

7.1.4

Tanθ =  VC HC

= 249,77

593,66

593,66N



(3)

θ     = 22,820                                                                 

E   = 644,06 N at 22,820 south of east  

OR

= 22

0 49

minutes

south of east

(3)

7.2         Stress and Strain

Stress = Pa Diameter = m Force = N

Force

Stress = force area

Force = Stress × Area                                 

2

Force

= 3500 000 × π × 0,025                    

4

Force

= 3,5×106

× 4,90873852 x 10−4              

=1718,06 N

Force

= 1,72 kN                                               

(4)

7.3         Stress and Strain

A = Limit of proportionality 

B = Elastic limit 

C = Yield point 

D = Maximum stress 

E = Break stress / Break point                                                                                             (5)

| 7.4 | Reactions Taking moments around A = (255 x 1,125) + (800 x 3,25) = (B x 7,75) + (350 x 1) 286,88 + 2600 = 7,75B + 350 B = 2536,88/7,75 B = 327,34 N Taking moments around B = A x 7,75 = (800 x 4,5) + (255 x 6,625) + (350 x 8,75) A x 7,75 = 3600 + 1689,38 + 3062,5 A = 8351,88/7,75 A = 1077,66 N | (6) |
| --- | --- | --- |
|  |  | [30] |

| 8.7 | Grease • Grease has a very high viscosity to ensure that it coats and sticks to the bearing surface it is lubricating. • To reduce rust  • To reduce noise  • Helps cool the bearings  • Increases the lifespan of the bearings  • Reduces friction  | (2) |
| --- | --- | --- |
|  |  | [15] |

QUESTION 8:  MAINTENANCE

| 8.1 | Viscosity To ensure that the gears are well coated with oil and do not lose the barrier of lubrication between them.  | (2) |
| --- | --- | --- |
| 8.2 | Reason using SAE20W50 This to ensure that the oil is able to satisfy the operational requirements over a range of temperature from start-up to running hot.   | (2) |
| 8.3 | Pour point Pour point is the lowest temperature at which a liquid remains pourable.  | (1) |
| 8.4 | Maintain cutting fluid |  |

•       Avoid contamination of  the  cutting fluid by draining and  regularly

replacing it. 

•       Always clean the machine's splash tray of metal cutting after use. 

•       Regularly wipe cutting fluid splashes of machine parts. 

•	Ensure that the sump is topped up from time to time and check that there is sufficient flow of cutting fluid to the cutting tool. 

•       Filter oil on a regular basis

| 8.5 | (Any 3 x 1) Belt drive maintenance | (3) |
| --- | --- | --- |
|  | Belt tends to stretch with prolonged use therefore they will need to be |  |
|  | tightened periodically and checked for correct alignment. |  |
|  | To transmit maximum torque without slippage | (2) |
| 8.6 | Reason skimming flywheel |  |
|  | The clutch plate presses against the flywheel. Due to friction between the clutch and flywheel it creates grooves/cracks in the flywheel. The grooves will need to be removed by a precision machining process known as |  |
|  | skimming before the new clutch plate is fitted. |  |
|  | To ensure that the co-efficient of friction the surfaces are at its maximum.  |  |
|  | To reduce wear and protect the new clutch plate.  | (3) |

•       Ensure that the correct soluble oil to water ratio is correct 

QUESTION 9:  SYSTEMS AND CONTROL

9.1         Gear drives

9.1.1      Number of teeth idler

NA × TA = NB × TB

N  × T

B

T  =    A         A                                                

NB

=  500 × 46                              

1000

= 23 teeth                               

(3)

9.1.2      Rotation frequency of the output shaft

NB × TB = NC × TC

NA × TA = NC × TC

C

N  = NB × TB                     

TC                                             OR

= 1000 × 23         

60

= 383,33 r/min    

N  = NA × TA                    

C

TC

=  500 × 46           

60

= 383,33 r / min   

(3)

9.2         Pulley Drives

9.2.1      Diameter of the driven pulley

N1 × D1 = N2 × D2

2

D  = N1 × D1                                                  

N2

=  7,2 × 600                             

12

= 360 mm                              

(3)

9.2.2      Power transmitted:

P = (T1 − T2 )πDn

T1 = 2,5

T2

P = (300 −120) π × 0,6 × 7,2               

= 2 442,9 Watts



= 2,44 kW

T  = 300      

2      2,5

= 120 N

OR

P = (T1 − T2 )πdn

P = (300 −120) π × 0,36 ×12       

= 2 442,9 Watts                      

T1 = 2,5           

T2

= 2,44 kW

T  = 300

2      2,5

= 120 N

(3)

9.3         Hydraulics

9.3.1      Fluid pressure

π

2

A B = D

π

4                                                 

2

=     x 0,076

4                                            

= 4,536459792 ×10-3 m2

B

P	=  F   A B



=           4000           Pa

4,536459792 ×10-3

= 881744,837 Pa                             

= 881,74 kPa

(4)

9.3.2      Diameter of piston A

PA   = PB

B

P    =  FA

A A

A

A    = FA

PB

A    =            140 N

A       881744,837N/m2                         

A

A    = 1,5877609 × 10 − 4

A

A    = 1,59 × 10 − 4                                         

πD2

=                                                              

4

D   =      A A × 4

π



1,59 × 10 − 4 × 4

=

π

= 0,0142182 m

= 14,22 mm                            

(5)

9.4         Traction Control

•	Prevent wheel from spinningif the torque transmitted to any other wheel which exceeds the minimum traction

•        Safety feature 

(2)

| 9.5 | Air Bags It is seen as a passive safety feature because the driver and passengers in the vehicle do not need to activate the air bags or do anything to be protected by air bags.  | (2) |
| --- | --- | --- |
|  |  | [25] |

QUESTION 10:  TURBINES

10.1       Reaction Turbine

•       Francis

•       Kaplan 

•       Tyson

•       Gorlov 

(Any 2 x 1)              (2)

10.2      Impulse Turbine

•       Impulse turbine changes the velocity of a water jet. 

•	The jet pushes on the turbine's curved blades which changes the direction of the flow

•	The resulting change in momentum (impulse) causes a force on the turbine blades. 

•	Since the turbine is spinning the force acts through a distance and the diverted water flow is left with diminished energy. 

•	Prior to hitting the turbine blades the water's pressure is converted to kinetic energy by a nozzle and focused on the turbine. 

•       No pressure change occurs at the turbine blades.                                              (6)

10.3       Control of speed of steam turbine

To prevent the turbine rotor leading to an over-speed trip. This causes the

nozzle valves that control the flow of steam to the turbine to close.                          (2)

10.4      Advantages of gas turbine

•       Smooth running due to absence of reciprocating parts. 

•	No rubbing parts such as piston so that internal friction and wear are almost eliminated. 

•       Easy starting. 

•       Can use wide range of fuels. 

•       No water cooling system required. 

•       Non-poisonous exhaust giving very little trouble with pollution

•       Require little routine maintenance. 

•	Very   high   power-to-weight   ratio,   compared   to   reciprocating engines.

•	Moves  in  one  direction  only,  with  far  less  vibration  than  a reciprocating engine. 

•       Low operating pressures. 

•       High operation speeds.

•       Low lubricating oil cost and consumption. 

(Any 3 x 1)              (3)

10.5      Auxiliary power units

•       To supply auxiliary power for larger machines. 

•       To supply compressed air for aircraft ventilation. 

•	Start power for larger jet engines, electrical and hydraulic power units.

(Any 2 x 1)              (2)

10.6      Purpose of supercharger

•	To fill the cylinder with an increased pressure that is higher than atmospheric pressure. 

•        To increase the compression pressure in the cylinder. 

•        To increase the volumetric efficiency of the engine.

•        To produce more engine power

•        Eliminates power loss above sea level

(Any 2 x 1)              (2)

10.7       High altitude

At high altitude less oxygen is available for combustion.

Loss of power                                                                                                                      (2)

| 10.8 | Advantage turbocharger Uses the exhaust gases to operate the turbo charger. No loss of power - needed to drive supercharger | (1) |
| --- | --- | --- |
|  |  | [20] |

TOTAL:            200