GRAAD 12

NATIONAL SENIOR CERTIFICATE/

NASIONALE SENIOR SERTIFIKAAT

GRADE/GRAAD 12

MECHANICAL TECHNOLOGY/MEGANIESE TEGNOLOGIE

FEBRUARY/MARCH/FEBRUARIE/MAART 2015

MARKS/PUNTE:  200

TIME/TYD:  3 hours/uur

This question paper consists of 14 pages and a 4-page formula sheet.

Hierdie vraestel bestaan uit 14 bladsye en 'n 4 bladsy-formuleblad.

3

Copyright reserved                                                                                                                  Please turn over

Kopiereg voorbehou                                                                                                             Blaai om asseblief

Meganiese Tegnologie                                                 2                                                  DBE/Feb.–Mrt. 2015

NSS

INSTRUKSIES EN INLIGTING

1.           Skryf jou sentrumnommer en eksamennommer in die spasies wat op die

ANTWOORDEBOEK verskaf word.

2.           Lees AL die vrae aandagtig deur.

3.           Beantwoord AL die vrae.

4.	Nommer die antwoorde korrek volgens die nommeringstelsel wat in hierdie vraestel gebruik is.

5.           Begin ELKE vraag op 'n NUWE bladsy.

6.           Toon  ALLE  berekeninge  en  eenhede.  Rond  finale  antwoorde  tot  TWEE

desimale plekke af.

7.           Jy   mag   'n   nieprogrammeerbare/wetenskaplike   sakrekenaar   en   teken-/

wiskundige instrumente gebruik.

8.           Die waarde van gravitasiekrag moet as 10 m/s2 geneem word.

9.           Alle afmetings is in millimeter, tensy anders in die vraag genoem word.

10.         Skryf netjies en leesbaar.

11.         'n Formuleblad verskyn aan die einde van die vraestel.

12.         Gebruik die kriteria hieronder om jou met die beplanning van jou tyd te help.

| VRAAG | INHOUD | PUNTE | TYD |
| --- | --- | --- | --- |
| 1 | Meervoudigekeuse-vrae | 20 | 15 minute |
| 2 | Veiligheid | 10 | 10 minute |
| 3 | Gereedskap en Toerusting | 12 | 10 minute |
| 4 | Materiale | 13 | 10 minute |
| 5 | Terminologie | 30 | 20 minute |
| 6 | Hegtingsmetodes | 25 | 25 minute |
| 7 | Kragte | 30 | 30 minute |
| 8 | Instandhouding | 15 | 15 minute |
| 9 | Stelsels en Beheer | 25 | 25 minute |
| 10 | Turbines | 20 | 20 minute |
| TOTAAL |  | 200 | 180 minute |

Mechanical Technology                                               2                                                 DBE/Feb.–Mar. 2015

NSC

INSTRUCTIONS AND INFORMATION

1.	Write your centre number and examination number in the spaces provided on the ANSWER BOOK.

2.           Read ALL the questions carefully.

3.           Answer ALL the questions.

4.	Number the answers correctly according to the numbering system used in this question paper.

5.           Start EACH question on a NEW page.

6.	Show ALL calculations and units. Round off final answers to TWO decimal places.

7.           You   may   use   a   non-programmable/scientific   calculator   and   drawing/

mathematical instruments.

8.           The value of gravitational force should be taken as 10 m/s2.

9.           All dimensions are in millimetres, unless stated otherwise in the question.

10.         Write neatly and legibly.

11.         A formula sheet appears at the end of the question paper.

12.         Use the criteria below to assist you in managing your time.

| QUESTION | CONTENT | MARKS | TIME |
| --- | --- | --- | --- |
| 1 | Multiple-choice questions | 20 | 15 minutes |
| 2 | Safety | 10 | 10 minutes |
| 3 | Tools and Equipment | 12 | 10 minutes |
| 4 | Materials | 13 | 10 minutes |
| 5 | Terminology | 30 | 20 minutes |
| 6 | Joining Methods | 25 | 25 minutes |
| 7 | Forces | 30 | 30 minutes |
| 8 | Maintenance | 15 | 15 minutes |
| 9 | Systems and Control | 25 | 25 minutes |
| 10 | Turbines | 20 | 20 minutes |
| TOTAL |  | 200 | 180 minutes |

Meganiese Tegnologie                                                 3                                                  DBE/Feb.–Mrt. 2015

NSS

VRAAG 1:  MEERVOUDIGEKEUSE-VRAE

Verskeie opsies word as moontlike antwoorde op die volgende vrae gegee. Kies die antwoord en skryf slegs die letter (A–D) langs die vraagnommer (1.1–1.20) in die ANTWOORDEBOEK neer, byvoorbeeld 1.21  A.

1.1         Watter   veiligheidsmaatreël   is   op   die   MIG/MAGS-sweisproses    van toepassing?

A	Gebruik altyd 'n vonkaansteker om die brander aan die brand te steek en nie 'n vuurhoutjie of 'n sigaretaansteker nie.

B      Die sweiser is ten volle geïsoleer met stewels en handskoene. C     Maak die silinderkleppe vinnig oop.

D     Maak voorsiening vir suurstof- en asetileenlekke.                                          (1)

1.2         Watter hardheidstoetser gebruik 'n staalbal om die hardheid van staal te bepaal?

A        Vickers-toetser

B        Rockwell-toetser

C        Victor-toetser

D        Brinell-toetser                                                                                                (1)

1.3       'n Gasanaliseerder word tydens die brandstofmengsel-instelling van 'n motorenjin  gebruik.  Watter  EEN  van  die  volgende  is  die  KORREKTE oorsaak van 'n hoë koolstofmonoksiedlesing?

A      Lae kompressie

B      Geslyte kleppe

C      'n Verstopte lugfilter

D      Geslyte suierringe                                                                                          (1)

1.4         Die funksie van die trektoetser:

A	Om die drukspanning en breekstootspanning op 'n stuk materiaal te bepaal

B      Om 'n trekspanning op 'n ondersteunde balk te demonstreer

C     Om die defleksie van 'n eenvoudig ondersteunde balk te demonstreer

D     Om die trekspanning, breektrekspanning en persentasie verlenging op

'n stuk materiaal te bepaal                                                                              (1)

1.5        Wanneer koolstofstaal teen 'n eenvormige tempo verhit word, styg die temperatuur eweredig tot  700 °C. Die  temperatuur  bly dan  vir  'n  rukkie konstant. Hierdie punt staan as die … bekend

A      afkoelingspunt

B      smeltpunt

C     laer kritieke punt

D     verhittingspunt                                                                                                (1)

Mechanical Technology                                               3                                                 DBE/Feb.–Mar. 2015

NSC

QUESTION 1:  MULTIPLE-CHOICE QUESTIONS

Various options are given as possible answers to the following questions. Choose the answer and write only the letter (A–D) next to the question number (1.1–1.20) in the ANSWER BOOK, for example 1.21   A.

1.1         What safety precaution is applicable to the MIGS/MAGS welding process?

A      Always use a striker to light the torch and not a match or lighter.

B      The welder is completely insulated by means of boots and gloves. C     Open the cylinder valves quickly.

D     Allow for oxygen and acetylene leaks.                                                            (1)

1.2         Which hardness tester uses a steel ball to determine the hardness of steel?

A        Vickers tester

B        Rockwell tester

C        Victor tester

D        Brinell tester                                                                                                  (1)

1.3         The gas analyser is used during the fuel mixture setting of a motor-car engine. Which ONE of the following is the CORRECT reason for a high carbon-monoxide reading?

A      Low compression

B      Worn valves

C      A clogged air filter

D      Worn piston rings                                                                                           (1)

1.4         The function of the tensile tester:

| A | To determine the compressive stress and ultimate pushing stress on a |  |
| --- | --- | --- |
|  | piece of a material |  |
| B | To demonstrate a pulling stress on a supported beam |  |
| C | To demonstrate the deflection of a simply supported beam |  |
| D | To determine yield stress, ultimate pulling stress and percentage of |  |
|  | elongation on a piece of material | (1) |

1.5         When carbon steel is heated at a uniform rate, the temperature rises evenly to 700 °C. The temperature then remains constant for a while. This point is known as the … point.

A      decalescent

B      melting

C     lower critical

D     heating                                                                                                            (1)

Meganiese Tegnologie                                                 4                                                  DBE/Feb.–Mrt. 2015

NSS

1.6         Staal  kan  verhard  en  teen  'n  temperatuur  tussen  885 °C  en  925 °C uitgegloei word. Watter persentasie koolstofinhoud sal hierdie verharding en uitgloeiing toelaat? Gebruik  die  yster-koolstof-ewewigsdiagram  in FIGUUR 1.1 hieronder.

|  |  |  |  |  |  |  | AC3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AC3 |  |  |  |  |  |  |  |
| AC1 |  |  |  |  |  |  | AC1 |

1 000

900

800

720

Temperatuur – °C

700

600

0             0,2          0,4          0,6           0,8         1,0           1,2         1,4

% – Koolstofinhoud

FIGUUR 1.1

A      0,60% B      0,20% C     0,80%

D     0,40%                                                                                                              (1)

1.7         Noem die gereedskapstuk wat gebruik word om die snybeitel haaks met die as van die werkstuk op te stel wanneer 'n skroefdraad in die draaibank gesny word:

A      Skroefsteekmeter

B      Skroefdraad-ringmaat

C     Skroefdraadsetmaat

D     Skroefmaat                                                                                                      (1)

1.8         Wat is die standaardverhouding van 'n tapse spy?

A      1 in 50

B      1 in 100

C      1 in 150

D      1 in 75                                                                                                             (1)

Mechanical Technology                                               4                                                 DBE/Feb.–Mar. 2015

NSC

1.6         Steel   can   be   hardened   and   annealed   at   a   temperature   between

885 °C  and  925 °C.  What  percentage  carbon  content  will  allow  for  this hardening and annealing? Use the iron carbon equilibrium diagram in FIGURE 1.1 below.

|  |  |  |  |  |  |  | AC3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AC3 |  |  |  |  |  |  |  |
| AC1 |  |  |  |  |  |  | AC1 |

1 000

900

Temperature – °C

800

720

700

600

0             0,2          0,4          0,6           0,8         1,0           1,2         1,4

% – Carbon content

FIGURE 1.1

A      0,60% B      0,20% C     0,80%

D     0,40%                                                                                                              (1)

1.7         Name the tool that is used to set the cutting tool perpendicular to the axis of the work piece when cutting a screw thread on the lathe:

A      Screw-pitch gauge

B      Screw-thread ring gauge

C     Centre gauge

D     Screw gauge                                                                                                   (1)

1.8         What is the standard ratio of a taper key?

A      1 in 50

B      1 in 100

C      1 in 150

D      1 in 75                                                                                                             (1)

Meganiese Tegnologie                                                 5                                                  DBE/Feb.–Mrt. 2015

NSS

1.9         Watter defek kom as 'n groef direk langs die rande van 'n sweislas in die moeder-/basismetaal voor?

A      Insnyding

B      Slakinsluiting

C     Poreusheid

D     Onvolledige indringing                                                                                    (1)

1.10       Watter EEN van die volgende is 'n voorbeeld van 'n destruktiewe toets?

A      X-straaltoets

B      Kleurstofdeurdringingstoets

C     Ultrasoniese toets

D     Buigtoets                                                                                                         (1)

1.11       Drukspanning kan as 'n interne krag in materiaal, wat weerstand bied teen

'n …, gedefinieer word.

A      skuiflas B      treklas C      druklas

D      lineêre las                                                                                                       (1)

1.12       Watter EEN van die volgende stellings omskryf Pascal se wet?

| A | Die oppervlakte is omgekeerd eweredig aan die druk daarop |  |
| --- | --- | --- |
|  | uitgeoefen, indien die temperatuur konstant bly. |  |
| B | Die druk uitgeoefen op die oppervlak van 'n vloeistof in 'n geslote |  |
|  | hidrouliese stelsel word in gelyke mate in alle rigtings oorgedra. |  |
| C | Die druk is eweredig aan die volume, indien die temperatuur konstant |  |
|  | bly. |  |
| D | Die volume is omgekeerd eweredig aan die druk daarop uitgeoefen, |  |
|  | indien die temperatuur verhoog. | (1) |

1.13        Die volgende stelling beskryf 'n voordeel van 'n bandaandrywingstelsel in vergelyking met 'n rataandrywingstelsel:

A      Sterker

B      Het geen smering nodig nie

C     Verander rigting

D     Meer duursaam                                                                                               (1)

Mechanical Technology                                               5                                                 DBE/Feb.–Mar. 2015

NSC

1.9         What defect appears as a groove in the parent metal, directly along the edges of the weld?

A      Undercutting

B      Slag inclusion

C     Porosity

D     Incomplete penetration                                                                                   (1)

1.10       Which ONE of the following is an example of a destructive test?

A      X-ray test

B      Liquid dye penetration test

C     Ultrasonic test

D     Bend test                                                                                                         (1)

1.11       Compressive stress can be defined as an internal force in material that

provides resistance against a … load.

A      shearing

B      tensile

C      compressive

D      linear                                                                                                               (1)

1.12       Which ONE of the following statements describes Pascal's law?

| A | The area is inversely proportional to the pressure on it if the |  |
| --- | --- | --- |
|  | temperature remains constant. |  |
| B | The pressure exerted on the surface of the liquid in a closed hydraulic |  |
|  | system is transmitted equally in all directions. |  |
| C | The pressure is proportional to the volume if the temperature remains |  |
|  | constant. |  |
| D | The volume is inversely proportional to the pressure on it if the |  |
|  | temperature increases. | (1) |

1.13        The following statement describes an advantage of a belt-drive system compared to a gear-drive system:

A      Stronger

B      Needs no lubrication

C     Changes direction

D     More durable                                                                                                   (1)

Meganiese Tegnologie                                                 6                                                  DBE/Feb.–Mrt. 2015

NSS

1.14       Bereken die vormverandering wanneer 'n trekkrag 'n spanning van 6 MPa in

'n werkstuk veroorsaak. Die materiaal het 'n elastisiteitsmodule van 3 GPa:

A      2 x 103

B      500

C     1,8 x 109

D     2 x 10-3                                                                                                                                                                  (1)

1.15       Wat is die samestelling van snyvloeistof?

A      Oplosbare olie en water

B      Ghries en water

C     Enjinolie en water

D     Masjienolie en water                                                                                       (1)

1.16       Waarvoor staan die afkorting EBE ('ECU') by voertuigbeheerstelsels?

A      Ekonomiese beheereenheid B      Elektroniese beheereenheid C     Elektriese beheereenheid

D     Enjinbeheereenheid                                                                                        (1)

1.17       Hoe word die superaanjaer aangedryf?

A      Hidrouliese aandrywing

B      Gasaandrywing

C      Pneumatiese aandrywing

D      Meganiese aandrywing                                                                                   (1)

1.18        Tydens 'n gasturbinetoepassing word die hulpkrageenheid beskryf as 'n …

gasturbine ontwerp vir hulpkrag.

A     groot

B     mediumgrootte

C     groter

D     klein                                                                                                                 (1)

1.19       Krag is 'n vektoreenheid wat deur … gekenmerk word.

A      slegs grootte

B      slegs rigting

C     grootte en rigting

D     volume en rigting                                                                                            (1)

Mechanical Technology                                               6                                                 DBE/Feb.–Mar. 2015

NSC

1.14       Calculate the strain when a tensile force causes a stress of 6 MPa in a work piece. The material has an elasticity module of 3 GPa:

A      2 x 103

B      500

C     1,8 x 109

D     2 x 10-3                                                                                                                                                                  (1)

1.15       What is the composition of cutting fluid?

A      Soluble oil and water

B      Grease and water

C     Engine oil and water

D     Machine oil and water                                                                                     (1)

1.16       What  does  the  abbreviation  ECU  stand  for  in  terms  of  the  vehicle management system?

A      Economical control unit

B      Electronic control unit

C      Electricity control unit

D      Engine control unit                                                                                          (1)

1.17       How is the supercharger driven?

A      Hydraulically driven

B      Gas driven

C      Pneumatically driven

D      Mechanically driven                                                                                        (1)

1.18        During  a  gas  turbine  application  the  auxiliary  power  unit  is  described

as … gas turbine designed for auxiliary power.

A     a large

B     a medium

C     a larger

D     a small                                                                                                             (1)

1.19       Force is a vector unit which is recognised by …

A      magnitude only. B      direction only.

C     magnitude and direction.

D     volume and direction.                                                                                     (1)

Meganiese Tegnologie                                                 7                                                  DBE/Feb.–Mrt. 2015

NSS

1.20       Wat stel punt D in FIGUUR 1.2 voor?

D

SPANNING

A B  C

E

VORMVERANDERING

FIGUUR 1.2

A      Eweredigheidsgrens

B      Maksimum vormverandering

C     Maksimum spanning

D     Elastisiteitsgrens                                                                                             (1)

[20]

Mechanical Technology                                               7                                                 DBE/Feb.–Mar. 2015

NSC

1.20       What does point D denote in FIGURE 1.2 below?

D

STRESS

A B  C

E

STRAIN

FIGURE 1.2

A      Limit of proportionality

B      Maximum strain

C     Maximum stress

D     Limit of elasticity                                                                                             (1)

[20]

Meganiese Tegnologie                                                 8                                                  DBE/Feb.–Mrt. 2015

NSS

VRAAG 2:  VEILIGHEID

2.1          Alle  persoonlike-  en  omgewingsveiligheidsmaatreëls  is  reeds  nagekom wanneer 'n vlakslyper gebruik word. Noem DRIE veiligheidsmaatreëls wat

slegs van toepassing is terwyl die vlakslyper gebruik word.                                   (3)

2.2          Gee TWEE redes waarom die drukmeter van 'n hidrouliese pers gereeld

getoets moet word.                                                                                                  (2)

2.3          Hoekom is dit belangrik dat die koperpunte van die puntsweiser tydens

gebruik konstant koel gehou moet word?                                                               (1)

2.4          Beskryf    die    posisie    van    die    volgende    met    betrekking    tot    die silinderlekkasietoets:

| 2.4.1 | Slag | (1) |
| --- | --- | --- |
| 2.4.2 | Suier | (1) |
| 2.4.3 | Kleppe | (1) |

2.5          Teen watter hoek tot die laer moet 'n laertrekker gebruik word?                            (1)

[10]

VRAAG 3:  GEREEDSKAP EN TOERUSTING

3.1          Verduidelik hoe 'n voltmeter en 'n ammeter aan 'n stroombaan gekoppel

word.                                                                                                                       (2)

3.2         Beskryf die doel van die volgende toetse:

|  | 3.2.1 Balkbuigtoets | (2) |
| --- | --- | --- |
|  | 3.2.2 Silinderlekkasietoets | (2) |
| 3.3 | Toe Johnny 'n droë kompressietoets uitgevoer het, het die toets getoon dat die eerste silinder 'n baie lae lesing het. Nadat 'n nat toets uitgevoer is, was die lesing hoër. Watter afleiding kan Johnny uit die toets maak? | (2) |
| 3.4 | FIGUUR 3.1 hieronder toon 'n kompressietoetser wat gebruik word om die druk van 'n silinder te toets. Benoem onderdeel A–D. |  |

A

B

C D

FIGUUR 3.1                                                             (4)

[12]

Mechanical Technology                                               8                                                 DBE/Feb.–Mar. 2015

NSC

QUESTION 2: SAFETY

2.1          All personal and environmental safety rules have already been taken care

|  | of when a surface grinder is used. State THREE safety rules which are only applicable while the surface grinder is being used. | (3) |
| --- | --- | --- |
| 2.2 | Give TWO reasons why the pressure gauge of a hydraulic press must be |  |
|  | tested regularly. | (2) |
| 2.3 | Why is it important to keep the copper tips of the spot welder constantly |  |
|  | cool during use? | (1) |
| 2.4 | Describe the position of the following regarding the cylinder leakage test: |  |
|  | 2.4.1 Stroke | (1) |
|  | 2.4.2 Piston | (1) |
|  | 2.4.3 Valves | (1) |
| 2.5 | At what angle to the bearing should a bearing puller be used? | (1) |
|  |  | [10] |
| QUESTION 3: TOOLS AND EQUIPMENT |  |  |
| 3.1 | Explain how a voltmeter and an ammeter are connected to a circuit. | (2) |
| 3.2 | Describe the purpose of the following tests: |  |
|  | 3.2.1 Beam-bending test | (2) |
|  | 3.2.2 Cylinder-leakage test | (2) |
| 3.3 | When Johnny conducted a dry compression test, the test indicated that the |  |
|  | first cylinder had a very low reading. After conducting a wet test, the reading |  |
|  | was higher. To what conclusion can Johnny come about the test? | (2) |
| 3.4 | FIGURE 3.1 below shows a compression tester that is used to check the |  |
|  | compression of a cylinder. Label parts A–D. |  |

A

B

C D

FIGURE 3.1                                                          (4)

[12]

Meganiese Tegnologie                                                 9                                                  DBE/Feb.–Mrt. 2015

NSS

VRAAG 4:  MATERIALE

4.1         Noem TWEE eienskappe van elk van die volgende mikroskopiese strukture van staal:

| 4.1.1 | Ferriet | (2) |
| --- | --- | --- |
| 4.1.2 | Perliet | (2) |

4.2         Bepaal  die  mikroskopiese  struktuur  wat  die  samestelling  van  yster  en koolstof (ysterkarbied) deur die analise van staal en gietyster  die beste beskryf.                                                                                                                    (2)

| KOOLSTOF- INHOUD | TIPIESE GEBRUIKE | HITTE- BEHANDELING | EIENSKAPPE |
| --- | --- | --- | --- |
| Laag 0,1–0,25% | 4.3.1 | Uitgloeiing | Sterk; duursaam |
| Medium 0,25–0,55% | Krukas; tange; skroewedraaiers | 4.3.2 | Taai; harde oppervlak |
| Hoog 0,55–1,00% | Snygereedskap; vere; hamers | Verharding | 4.3.3 |

4.3         Die   tabel   hieronder   dui   die   koolstofinhoud,   tipiese   gebruike,   hitte- behandeling   en   eienskappe   van   staal   aan.   Skryf   die   antwoord   op VRAAG 4.3.1, 4.3.2 en 4.3.3 in die ANTWOORDEBOEK neer.

(3)

4.4         Definieer   die   volgende   terme   met   verwysing   na   die   yster-koolstof- ewewigsdiagram:

4.4.1         Laer kritieke punt (AC1)                                                                          (2)

4.4.2         Kritieke temperatuur                                                                               (2)

[13] VRAAG 5:  TERMINOLOGIE

5.1         Verduidelik stapsgewys hoe 'n metrieke V-skroefdraad met 'n steek van

1,5 mm op 'n senterdraaibank gesny word.                                                           (11)

5.2         Bereken  die snydiepte  van  'n  metrieke  V-skroefdraad  met  'n  steek  van

2,5 mm wanneer die saamgestelde-beitelslee-metode gebruik word.                    (3)

5.3         Bereken die eenvoudige indeksering wat nodig is om 'n rat met 82 tande

te sny.                                                                                                                      (3)

5.4         Die lengte van 'n parallelspy is 102 mm. Bereken:

5.4.1         Die diameter van die as                                                                         (3)

5.4.2         Die wydte van die spy                                                                            (3)

5.4.3         Die dikte van die spy                                                                              (3)

5.5         Toon,  met  behulp  van  netjiese  benoemde  sketse,  die  verskil  tussen

opfreeswerk en klimfreeswerk aan.                                                                         (4)

Mechanical Technology                                               9                                                 DBE/Feb.–Mar. 2015

NSC

QUESTION 4:  MATERIALS

4.1         Name TWO characteristics of each of the following microscopic structures of steel:

| 4.1.1 |  | Ferrite | (2) |
| --- | --- | --- | --- |
| 4.1.2 |  | Pearlite | (2) |
| 4.2 Determine the microscopic structure that best describes the compound of |  |  |  |
|  | iron and carbon (iron carbide) by analysing steel and cast iron. |  | (2) |
| 4.3 | The table below indicates the carbon content, typical uses, heat treatment and properties of steel. Write your answers for QUESTIONS 4.3.1, 4.3.2 and 4.3.3 in the ANSWER BOOK. |  |  |

(3)

| CARBON CONTENT | TYPICAL USES | HEAT TREATMENT | PROPERTIES |
| --- | --- | --- | --- |
| Low 0,1–0,25% | 4.3.1 | Annealing | Strong; durable |
| Medium 0,25–0,55% | Crankshafts; pliers; screwdrivers | 4.3.2 | Tough; hard surface |
| High 0,55–1,00% | Cutting tools; springs; hammers | Hardening | 4.3.3 |

4.4       Define the terms below with reference to the iron-carbon equilibrium diagram:

| 4.4.1 Lower critical point (AC1) | (2) |
| --- | --- |
| 4.4.2 Critical temperature | (2) [13] |
| QUESTION 5: TERMINOLOGY |  |
| 5.1 Explain step by step how a metric V-screw thread with a pitch of 1,5 mm is cut on the centre lathe. | (11) |
| 5.2 Calculate the cutting depth of a metric V-screw thread with a pitch of 2,5 mm using the compound slide method. | (3) |
| 5.3 Calculate the simple indexing needed to cut a gear with 82 teeth. | (3) |
| 5.4 The length of a parallel key is 102 mm. Calculate: |  |
| 5.4.1 The diameter of the shaft | (3) |
| 5.4.2 The width of the key | (3) |
| 5.4.3 The thickness of the key | (3) |

5.5         Show, by means of neat labelled sketches, the difference between upcut

milling and downcut milling.                                                                                     (4)

Meganiese Tegnologie                                                10                                                 DBE/Feb.–Mrt. 2015

NSS

VRAAG 6:  HEGTINGSMETODES

6.1         FIGUUR 6.1 toon 'n sweismasjien met verskillende toebehore.

FIGUUR 6.1

|  | 6.1.1 Identifiseer die sweismasjien in FIGUUR 6.1. | (1) |
| --- | --- | --- |
|  | 6.1.2 Benoem onderdeel A–G in FIGUUR 6.1. | (7) |
| 6.2 | Verduidelik die werkbeginsel van die X-straaltoetstoerusting soos van toepassing op 'n sweislas. | (6) |
| 6.3 | Noem DRIE voordele van afgeskermde metaalboogsweiswerk (MIGS/MAGS). | (3) |
| 6.4 | Wat is die doel van 'n buigtoets? | (2) |
| 6.5 | Noem TWEE oorsake van die volgende sweisdefekte: |  |
|  | 6.5.1 Onvolledige indringing | (2) |
|  | 6.5.2 Sweiskraters | (2) |

6.6         Watter  TWEE  aspekte  met  betrekking  tot  sweistegnieke  moet  tydens boogsweis in gedagte gehou word?                                                                        (2)

[25]

Mechanical Technology                                              10                                                DBE/Feb.–Mar. 2015

NSC

QUESTION 6: JOINING METHODS

6.1         FIGURE 6.1 shows a welding machine with different attachments.

FIGURE 6.1

|  | 6.1.1 Identify the welding machine in FIGURE 6.1. | (1) |
| --- | --- | --- |
|  | 6.1.2 Label parts A–G in FIGURE 6.1. | (7) |
| 6.2 | Explain the operating principle of the X-ray testing equipment as applicable to a welded joint. | (6) |
| 6.3 | State THREE advantages of metal-arc shielded welding (MIGS/MAGS). | (3) |
| 6.4 | What is the purpose of a bend test? | (2) |
| 6.5 | State TWO causes of the following welding defects: |  |
|  | 6.5.1 Incomplete penetration | (2) |
|  | 6.5.2 Welding craters | (2) |
| 6.6 | Which TWO aspects regarding welding techniques should be kept in mind during arc welding? | (2) |
|  |  | [25] |

Meganiese Tegnologie                                                11                                                 DBE/Feb.–Mrt. 2015

NSS

VRAAG 7:  KRAGTE

7.1          Vier kragte van 150 N, 210 N, 250 N en 360 N onderskeidelik, soos getoon in FIGUUR 7.1 hieronder, werk op dieselfde punt in. Bereken die grootte en die rigting van die ewewigskrag vir hierdie stelsel van kragte.

210 N

110°

45°

250 N

360 N

|  | 150 N |  |
| --- | --- | --- |
|  | FIGUUR 7.1 | (15) |
| 7.2 | 'n Vierkantige staalstaaf, met 100 mm x 100 mm-sye, word aan 'n drukkrag van 80 kN onderwerp. Bepaal, deur middel van berekeninge, die spanning in die materiaal. | (5) |
| 7.3 | Definieer Hooke se wet. | (3) |
| 7.4 | FIGUUR 7.2 hieronder toon 'n eenvormige balk wat deur twee vertikale stutte, A en B, ondersteun word. Twee vertikale puntbelastings word op die balk uitgeoefen, asook 'n eenvormige verspreide belasting van 50 N/m, oor die hele linkerhelfte van die balk. |  |

Bepaal, deur middel van  berekeninge,  die groottes van die reaksies in stut A en B.

50 N/m eenvormige verspreide belasting

400 N        600 N

1,5 m                 5 m

2 m                 4,5 m

A                                                                                       B

FIGUUR 7.2                                                          (7)

[30]

Mechanical Technology                                              11                                                DBE/Feb.–Mar. 2015

NSC

QUESTION 7:  FORCES

7.1          Four forces of 150 N, 210 N, 250 N and 360 N respectively, as shown in FIGURE 7.1 below, act on the same point. Calculate the magnitude and direction of the equilibrant for this system of forces.

210 N

110°

45°

250 N

360 N

| 7.2 7.3 | 150 N |  |
| --- | --- | --- |
|  | FIGURE 7.1 | (15) |
|  | A square steel bar with 100 mm x 100 mm sides is subjected to a compressive force of 80 kN. Determine, by means of calculations, the stress in the material. | (5) |
|  | Define Hooke's law. | (3) |
| 7.4 | FIGURE 7.2 below shows a uniform beam that is supported by two vertical |  |

supports, A and B. Two vertical point loads are exerted onto the beam, as

well as a uniformly distributed load of 50 N/m, over the total left half of the beam.

Determine, by means of calculations, the magnitudes of the reactions in supports A and B.

50 N/m uniformly distributed load

400 N        600 N

1,5 m                 5 m

2 m                 4,5 m

A                                                                                       B

FIGURE 7.2                                                          (7)

[30]

Meganiese Tegnologie                                                12                                                 DBE/Feb.–Mrt. 2015

NSS

VRAAG 8:  INSTANDHOUDING

| 8.1 | Noem TWEE voordele van 'n snyvloeistof. |  |  | (2) |
| --- | --- | --- | --- | --- |
| 8.2 | Definieer voorkomende instandhouding. |  |  | (1) |
| 8.3 | Motorvervaardigers beveel aan dat die tydreëlketting |  | van 'n voertuigenjin |  |
| elke 90 000 km vervang moet word. Beantwoord die vrae wat volg. |  |  |  |  |
| 8.3.1 |  | Gee TWEE redes waarom 'n kettingaandrywing bo 'n band- aandrywing verkies word. |  | (2) |
| 8.3.2 |  | Gee TWEE redes waarom 'n gerekte ketting vervang moet word. |  | (2) |
| 8.3.3 |  | Verduidelik puntsgewys hoe jy die tydreëlketting van 'n enjin sal verwyder en vervang. |  | (6) |

8.4          Waarom is dit wenslik dat enjinolie 'n hoë flitspunt moet hê?                                 (2)

[15] VRAAG 9:  STELSELS EN BEHEER

9.1         FIGUUR 9.1 hieronder toon 'n ratstelsel wat gebruik word om 'n hysmasjien te beheer. Die dryfrat het 50 tande en roteer teen 660 r/min. Die tussenrat wat gebruik word om die draairigting te verander, draai teen 1 000 r/min. Die gedrewe rat het 60 tande.

Rat A

Rat-A

Rat B

Rat-B

Rat C

Rat-C

Dryfrat

Tussenrat

FIGUUR 9.1

Gedrewe rat

Bepaal, deur middel van berekeninge:

| 9.1.1 | Die getal tande van die tussenrat |  | (3) |
| --- | --- | --- | --- |
| 9.1.2 | Die rotasiefrekwensie van die gedrewe sekonde | rat in omwentelinge per | (3) |

Mechanical Technology                                              12                                                DBE/Feb.–Mar. 2015

NSC

QUESTION 8:  MAINTENANCE

| 8.1 | State TWO advantages of a cutting fluid. |  |  | (2) |
| --- | --- | --- | --- | --- |
| 8.2 | Define preventive maintenance. |  |  | (1) |
| 8.3 | Car manufacturers recommend that the |  | timing chain of a car engine be |  |
| replaced every 90 000 km. Answer the questions that follow. |  |  |  |  |
| 8.3.1 |  | Give TWO reasons why a chain drive is preferred to a belt drive. |  | (2) |
| 8.3.2 |  | Give TWO reasons why a stretched chain has to be replaced. |  | (2) |
| 8.3.3 |  | Explain in point form how you would remove and replace the timing chain of an engine. |  | (6) |

8.4          Why is it desirable for engine oil to have a high flash point?                                  (2)

[15] QUESTION 9: SYSTEMS AND CONTROL

9.1         FIGURE 9.1 below shows a gear system used to control a hoisting machine.

The driver gear has 50 teeth and rotates at 660 r/min. The idler gear that is used to change the direction, rotates at 1 000 r/min. The driven gear has

60 teeth.

Gear A

Gear B

Gear C

Driver gear

Idler gear

Driven gear

FIGURE 9.1

Determine by means of calculations:

| 9.1.1 | The number of teeth on the idler gear |  | (3) |
| --- | --- | --- | --- |
| 9.1.2 | The rotation frequency of the driven second | gear in revolutions per | (3) |

Meganiese Tegnologie                                                13                                                 DBE/Feb.–Mrt. 2015

NSS

9.2         Die dryfkatrol van 'n bandaandrywingstelsel roteer teen 1 640 r/min. Die dryfkatrol het 'n diameter van 175 mm en die gedrewe katrol se diameter is

80 mm. Die banddikte is 12 mm.

Neem die banddikte in aanmerking en bepaal, deur middel van berekeninge:

9.2.1         Die rotasiefrekwensie van die gedrewe katrol in revolusies per sekonde                                                                                                   (3)

9.2.2         Die bandspoed van die stelsel                                                                (3)

9.3         'n Hidrouliese stelsel word gebruik om skrootmetaal vir herwinning saam te pers. Die spesifikasies van die stelsel word diagrammaties in FIGUUR 9.2 hieronder voorgestel.

Ø 38 mm

240 N

Ø 150 mm

Las = ? N

Suier A

Suier B

FIGUUR 9.2

Bepaal, deur middel van berekeninge:

9.3.1         Die   vloeistofdruk   in   die   hidrouliese   stelsel   wanneer  dit   in

ewewig is                                                                                                 (3)

9.3.2         Die krag uitgeoefen deur suier B                                                            (4)

9.4        Beskryf die doel van die voertuigenjin-beheerstelsel.                                              (4)

9.5        Beskryf die doel van die sluitweerremstelsel (ABS).                                                (2)

[25]

Mechanical Technology                                              13                                                DBE/Feb.–Mar. 2015

NSC

9.2         The drive pulley of a belt-drive system rotates at 1 640 r/min. The drive pulley has a diameter of 175 mm and the driven pulley a diameter of 80 mm. The belt thickness is 12 mm.

Take  the  belt  thickness  into  consideration  and  determine,  by  means  of calculations:

9.2.1         The  rotation  frequency  of  the  driven  pulley  in  revolutions  per

second                                                                                                     (3)

9.2.2         The belt speed of the system                                                                  (3)

9.3         A hydraulic system is used to compress scrap metal for recycling. The specifications of the system are presented diagrammatically in FIGURE 9.2.

Ø 38 mm

240 N

Ø 150 mm

Load = ? N

Piston A

Piston B

FIGURE 9.2

Determine, by means of calculations:

|  | 9.3.1 The fluid pressure in the hydraulic system while in equilibrium | (3) |
| --- | --- | --- |
|  | 9.3.2 The force exerted by piston B | (4) |
| 9.4 | Describe the purpose of the vehicle-engine management system. | (4) |
| 9.5 | Describe the purpose of the anti-lock brake system (ABS). | (2) [25] |

Meganiese Tegnologie                                                14                                                 DBE/Feb.–Mrt. 2015

NSS

VRAAG 10:  TURBINES

| 10.1 | Noem TWEE positiewe invloede wat 'n waterturbine op die omgewing en die samelewing sal hê. | (2) |
| --- | --- | --- |
| 10.2 | Definieer die volgende terme rakende 'n waterturbine: |  |
|  | 10.2.1 Soortlike spoed | (2) |
|  | 10.2.2 Vryloopspoed | (2) |
| 10.3 | Noem die funksie van 'n stoomturbine. | (2) |
| 10.4 | Noem DRIE tipes stoomturbines. | (3) |
| 10.5 | Wat is die voordeel van die gebruik van gasturbines op vlootvaartuie? | (2) |
| 10.6 | Definieer aanjagingsdruk. | (2) |
| 10.7 | Verduidelik puntsgewys die werking van 'n dubbelskroef-drukaanjaer ('supercharger') | (5) |
|  |  | [20] |
|  | TOTAAL: | 200 |

Mechanical Technology                                              14                                                DBE/Feb.–Mar. 2015

NSC

QUESTION 10: TURBINES

10.1      State TWO positive impacts a water turbine will have on the environment

and society.                                                                                                             (2)

| 10.2 | Define the following terms regarding a water turbine: |  |  |
| --- | --- | --- | --- |
|  | 10.2.1 Specific speed |  | (2) |
|  | 10.2.2 Free load speed/Runaway speed |  | (2) |
| 10.3 | State the function of a steam turbine. |  | (2) |
| 10.4 | Name THREE types of steam turbines. |  | (3) |
| 10.5 | What is the advantage of using gas turbines on naval vessels? |  | (2) |
| 10.6 | Define turbo boost. |  | (2) |
| 10.7 | Explain in point form the operation of a twin-screw supercharger. |  | (5) [20] |
|  |  | TOTAL: | 200 |

Meganiese Tegnologie                                                 1                                                  DBE/Feb.–Mrt. 2015

NSS

FORMULEBLAD VIR MEGANIESE TEGNOLOGIE – GRAAD 12

1.            BANDAANDRYWINGS

1.1

1.2

Bandspoed   πD N

60

Bandspoed =  π  ( D t )N

60

(t  banddikte)

1.3

Bandmassa  oppervlakte  lengte   digtheid

(A  dikte  wydte)

1.4

Spoedverhouding

   diameter van gedrewe katrol diameter van dryfkatrol

1.5

N1 D1   N2 D2

1.6

π(  D d )    ( D d )2 	 Oopbandlengte                                       2c

2                  4c

1.7

π(  D d )    ( D d ) 2

Gekruisteband lengte                                         2c

1.8

2                   4c

Drywing ( P )   2π NT

60

1.9

Verhouding tussen die stywe kant en slap kant  T1

T2

1.10

Drywing (P) 

(T1  T2  ) π  D N

60

waar T1   krag in die stywe kant

T2   krag in die slap kant

T1  T2   effektiewe krag (Te )

1.11

Wydte

                 T1

toelaatbar e trekkrag

2.            WRYWINGKOPPELAARS

2.1

Wringkrag ( T ) 

µWnR

waar

µ  wrywingskoëffisiën t

W  totale druk

n  getal wrywingsoppervlakke

R  effektiewe radius

2.2

Drywing ( P ) 

2π NT

60

Mechanical Technology                                               1                                                 DBE/Feb.–Mar. 2015

NSC

FORMULA SHEET FOR MECHANICAL TECHNOLOGY – GRADE 12

1.            BELT DRIVES

1.1

Belt speed

  πD N

60

1.2

Belt  speed    π ( D t )N

60

(t  belt thickness)

1.3

Belt  mass  area  length  density

(A  thickness  width)

1.4

Speed ratio  

diameter of driven pulley

diameter of

driver pulley

1.5

N1 D1   N2 D2

1.6

π(  D d )    ( D d ) 2

Open - belt length                                         2c

2                   4c

1.7

π(  D d )     ( D d )2   Crossed-belt length                                      2c

1.8

Power ( P ) 

2                 4c

2 π  N T

60

1.9

Ratio of  tight side to slack side  T1

T2

1.10

Power (P) 

(T1  T2  ) π  D N

60

where T1  

T2  

force in the tight side force in the slack side

Width

T1   T2   effective

                    T1

force (Te )

1.11

permissible tensile

force

2.            FRICTION CLUTCHES

2.1

Torque ( T

where

)  µWnR

µ  coefficien t of

friction

W  total

force

n  number of

friction surfaces

R  effective

radius

2.2

Power ( P ) =

2 π NT

60

Meganiese Tegnologie                                                 2                                                  DBE/Feb.–Mrt. 2015

NSS

3.            SPANNING EN VORMVERANDERING

3.1

Spanning       krag         of oppervlakte

( σ   F  ) A

Vormverandering ( 

)   verandering in lengte ( ΔL )

3.2

oorspronklike lengte ( L )

3.3

Young se modulus ( E )        spanning           of vormverandering

πD  2

(   σ    )

ε

π D 2 d 2 

3.4

Oppervlaktero n d e sta a f  

4

en  Oppervlaktero n d e p yp  

4

Oppervlaktevierka n tige sta a f

 L2

en    Oppervlaktevierka n tige  p yp

 L2   l 2

4.            HIDROULIKA

Druk ( P )        krag ( F  )

4.1

oppervlakte ( A )

4.2    Volume = dwarsdeursnee-oppervlakte  slaglengte ( l of s )

4.3    Arbeid verrig = krag  afstand

4.4

πD2

Oppervlakte 

4

4.5

F1     F2

A1          A2

5.            RATAANDRYWINGS

5.1

Drywing ( P ) 

2π NT

60

5.2

Ratverhouding   produk van die getal tande op gedrewe  ratte produk van die getal tande op dryfratte

N in set     produk van die getal tande op gedrewe ratte

5.3

Nuitset

produk van die getal tande op dryfratte

5.4

Wringkrag  krag  radius

5.5

Wringkrag oorgedra  ratverhouding  insetwringkrag

Mechanical Technology                                               2                                                 DBE/Feb.–Mar. 2015

NSC

3.            STRESS AND STRAIN

3.1

Stress   force area

or 	( σ   F  ) A

3.2

Strain ( ε )   change in length ( ΔL  )

original length ( L )

3.3

3.4

Young' s modulus ( E )   stress strain

D 2

or    (  σ   )

ε

D 2  d 2 

Area ro u n dbar  

4

and

Area ro u n dtu b e  

4

Area

sq u a rebar

 L2

and

Area

sq a u re tu b e

 L2   l 2

4.            HYDRAULICS

Pressure ( P )   force ( F )

4.1

area ( A )

4.2    Volume = cross-sectional area × stroke length ( l or s )

4.3

Work done  force   distance

4.4

Area 

D 2

4

4.5

F1     F2

A1         A2

5.            GEAR DRIVES

5.1

Power ( P ) 

2 π  N T

60

5.2

Gear ratio    product of  the number of  teeth on driven gears

product of the number of teeth on driving gears

N in p u t       product of  the number of  teeth on driven gears

5.3

N output

product of the number of teeth on driving gears

5.4

Torque  force  radius

5.5

Torque transmitted  gear ratio  input torque

Meganiese Tegnologie                                                 3                                                  DBE/Feb.–Mrt. 2015

NSS

5.6

Module ( m ) =  Steeksirkeldiameter ( SSD ) Getal tande ( T )

5.7

5.8

N1T1   N2T2

Steeksirkeldiameter  ( SSD )   sirkelsteek ( SS ) getal tande ( T )

π

5.9

Buitediame ter ( BD  ) = SSD + 2 module

5.10

Addendum ( a )  module ( m )

5.11

Dedendum ( b )  1,157 m

of         Dedendum ( b )  1,25 m

5.12

Snydiepte ( h )  2,157 m

of         Snydiepte ( h )  2,25 m

5.13

Vry ruimte ( c )  0,157 m

of         Vry ruimte ( c )  0,25 m

5.14

Sirkelsteek ( SS )  m  π

6.            KATROLAANDRYWINGS

6.1

6.2

N1 D1   N2 D2

Drywing ( P )   2π NT

60

6.3

Spoedverhouding

   diameter van gedrewe katrol diameter van dryfkatrol

7.            SPYE

7.1

Wydte

van

spy 

diameter

4

van as

7.2

Dikte  van spy 

diameter

6

van as

7.3

Lengte

van

spy  1,5  diameter

van  as

7.4

Tapsheid

van  spy  1 : 100

Mechanical Technology                                               3                                                 DBE/Feb.–Mar. 2015

NSC

Module  ( m )   pitch -circle diameter  ( PCD )

5.6

number of teeth

( T )

5.7

5.8

N1T1   N2T2

Pitch -circle diameter  ( PCD )   circular  pitch  ( CP ) number of teeth ( T )

π

5.9

Outside diameter ( OD )  PCD  2 module

5.10

Addendum ( a )  module ( m )

5.11

Dedendum ( b )  1,157  m

or         Dedendum ( b )  1,25 m

5.12

Cutting depth  ( h )  2,157 m

or        Cutting depth ( h )  2,25 m

5.13

Clearance ( c )  0,157 m

or        Clearance ( c )  0,25 m

5.14

Circular

pitch  ( CP )  m  π

6.            PULLEY DRIVES

6.1

6.2

N1 D1   N2 D2

Power ( P )   2 π  NT

60

Velocity

Ratio   diameter  of  driven pulley

6.3

diameter  of

driver

pulley

7.            KEYWAYS

7.1

Width of key 

diameter of shaft

4

7.2

Thickness of key 

diameter of shaft

6

7.3

Length  of the key  1,5  diameter of

shaft

7.4

Taper  of key  1 : 100

Meganiese Tegnologie                                                 4                                                  DBE/Feb.–Mrt. 2015

NSS

8.            HEFBOME

Meganiese voordeel (  MA )      las ( W  )

8.1

hyskrag ( F  )

8.2

Insetbeweging ( IM

)  hyskrag  afstand  beweeg  deur hyskrag

8.3

Uitsetbeweging ( OM

)  las  afstand  beweeg  deur las

8.4

Snelheidsverhouding ( VR

) =  insetbeweg ing

uitsetbewe ging

9.            SKROEFDRAAD

9.1

Steekdiameter  buitediameter  ½ steek

9.2

Steekomtrek  π  steekdiameter

9.3

Styging   steek  getal beginne

9.4

Hoogte

van

skroefdraad  0,866

  steek (P)

9.5

Diepte

van

skroefdraad  0,613

  steek (P)

9.6	Getal draaie   hoogte styging

10.          INDEKSERING

CINCINNATI-VERDEELKOPTABEL VIR GATSIRKELS IN PLAAT

| Gatsirkels |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kant 1 | 24 | 25 | 28 | 30 | 34 | 37 | 38 | 39 | 41 | 42 | 43 |
| Kant 2 | 46 | 47 | 49 | 51 | 53 | 54 | 57 | 58 | 59 | 62 | 66 |

Indeksering    40 n

Mechanical Technology                                               4                                                 DBE/Feb.–Mar. 2015

NSC

8.            LEVERS

Mechanical advantage ( MA )   load ( W )

8.1

effort ( F )

8.2

Input  movement ( IM  )  effort   distance moved by effort

8.3

Output  movement ( OM

)  load  distance moved by load

8.4

Velocity

ratio ( VR )   input  movement

output  movement

9.            SCREW THREADS

9.1

Pitch  diameter  outside  diameter  ½ pitch

9.2

Pitch  circumference  π  pitch diameter

9.3

Lead   pitch  number of

starts

9.4

Height  of

screw thread

 0,866   pitch ( P )

9.5

Depth  of screw thread

0,613  pitch

( P )

9.6

Number of turns  height lead

10.          INDEXING

CINCINNATI DIVIDING HEAD TABLE FOR HOLE CIRCLES IN PLATE

| Hole circles |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Side 1 | 24 | 25 | 28 | 30 | 34 | 37 | 38 | 39 | 41 | 42 | 43 |
| Side 2 | 46 | 47 | 49 | 51 | 53 | 54 | 57 | 58 | 59 | 62 | 66 |

Indexing    40 n