NATIONAL SENIOR CERTIFICATE

GRADE 12

MECHANICAL TECHNOLOGY FEBRUARY/MARCH 2015

MEMORANDUM

MARKS:  200

This memorandum consists of 18 pages.

QUESTION 1:  MULTIPLE-CHOICE QUESTIONS

1.1         B                                                                                                                                           (1)

1.2         D                                                                                                                                          (1)

1.3         C                                                                                                                                          (1)

1.4         D                                                                                                                                          (1)

1.5         A                                                                                                                                           (1)

1.6         B                                                                                                                                           (1)

1.7         C                                                                                                                                          (1)

1.8         B                                                                                                                                           (1)

1.9         A                                                                                                                                           (1)

1.10       D                                                                                                                                          (1)

1.11       C                                                                                                                                          (1)

1.12       B                                                                                                                                           (1)

1.13       B                                                                                                                                           (1)

1.14       D                                                                                                                                           (1)

1.15       A                                                                                                                                           (1)

1.16       B                                                                                                                                           (1)

1.17       D                                                                                                                                          (1)

1.18       D                                                                                                                                          (1)

1.19       B                                                                                                                                           (1)

1.20       C                                                                                                                                          (1)

[20]

QUESTION 2:  SAFETY

| 2.1 | Surface Grinder: • Make sure the sparks are of no danger to co-workers.  • Do not force the grinding wheel onto the material.  • Bring the grinding wheel slowly into contact with the material.  | (3) |
| --- | --- | --- |
| 2.2 | Pressure Gauge: |  |
|  | • To make sure there is no leakages.  |  |
|  | • To make sure that the readings are accurate.  | (2) |
| 2.3 | Spot Welding: To prevent the tips from overheating during operation.  | (1) |
| 2.4 | Cylinder Leakage test:: |  |
|  | 2.4.1 Stroke: The beginning of compression stroke  | (1) |
|  | 2.4.2 Piston: Bottom dead centre  | (1) |
|  | 2.4.3 Valves: Both valves are closed  | (1) |
| 2.5 | Bearing Puller: Perpendicular/90° to the bearing.  | (1) |
|  |  | [10] |

QUESTION 3: TOOLS AND EQUIPMENT

| 3.1 | Volt and ammeter: |  |
| --- | --- | --- |
|  | • Voltmeter: connected in parallel to a circuit.  |  |
|  | • Ammeter: connected in series to a circuit.  OR |  |
|  | • Credit should be given to the learner for the drawing illustrating the correct answer. | (2) |

3.2         Beam bending and cylinder leakage tests:

3.2.1         A  beam  bending  test  is  to  investigate  the  deflection    of

beams.                                                                                                               (2)

3.2.2         A cylinder leakage tester is to check whether gasses leak 

from the cylinders.                                                                                          (2)

3.3         Compression Test:

The rings are worn out.                                                                                                   (2)

3.4         Compression tester:

A – Spark plug adaptor 

B – Pressure gauge 

C – Pressure release valve 

D – Rubber pipe                                                                                                                            (4)

[12]

QUESTION 4:  MATERIALS

4.1        Properties of structures:

4.1.1       •       Soft 

•       Ductile 

•       Grey or white in colour 

(Any 2x1)          (2)

4.1.2       •      Ductile 

•      Hard 

•      Strong and tough 

•      Resistant to deformation 

(Any 2x1)          (2)

4.2        Cementite                                                                                                                         (2)

4.3        Classes of steel

4.3.1       Bolts, nuts, screws and rivets 

(Any 1x1)          (1)

4.3.2       Surface hardening (case hardened), hardening and tempering 

(Any 1x1)          (1)

4.3.3       Brittle, poor weldability 

(Any 1x1)          (1)

4.4        Definition:

4.4.1       Lower Critical point (AC1 ):

This is the lowest temperature to which steel must be heated to

be hardened.                                                                                                  (2)

4.4.2       Critical temperature:

It is the temperature where a structural change takes place.                (2)

[13]

QUESTION 5:  TERMINOLOGY

5.1          V-Screw thread cutting  :

•	Set up the work-piece in the lathe and turn the part to be threaded to the major diameter of the thread. 

•	Set the compound slide 30° to the right and set the tool up accurately in the post 

•     Set the quick-change gearbox for 1,5 mm pitch 

•	Start the lathe and set the cutting tool so that it just touches the work- piece. Set graduated dials to zero (cross feed and compound slide) 

•	Move cutting tool a short distance off end of work-piece and feed compound slide say 0,06 mm inwards. 

•	With lathe turning, engage half nuts at the correct line on the chasing dial, putting the first cut in progress 

•	Withdraw the cutting tool quickly at the end of the cut and disengage the half-nut lever. Return the carriage to the starting point of the thread. 

•	Stop the centre lathe and check with thread gauge to see if thread pitch is correct. 

•	Repeat with successive cuts until thread is complete. (Remember to bring cross-feed collar back to zero for each cut). 

•     Each successive cut is set by means of the compound slide. 

•      Check thread with ring gauge for correct fit.                                                         (11)

5.2          Cutting depth:

Cutting depth = 0,866 x P           

= 0,866 x 2,5        

= 2,17 mm            

(3)

5.3          Indexing:

Indexing = 40                                                            

n

= 40

82

= 20                                                             

41

No full turns. 20 holes on a 41 hole circle               

(3)

5.4         Key calculations:

5.4.1         Key length:

Length

Diameter

Diameter

Diameter

= 1.5 × Diameter             

=   L

1.5                               

= 102

1.5

= 68 mm                         

(3)

5.4.2         Key width:

Width

Width

= D                                     

4                                     

= 68

4

= 17 mm                              

(3)

5.4.3         Key thickness:

Thickness = D                               

6

= 68                              

6

Thickness =11.33 mm                   

(3)

5.5         Milling operations:

Up-cut milling:





(2)

Down-cut milling:





(2)

[30]

QUESTION 6: JOINING METHODS

6.1        Welding Machine:

6.1.1           MIGS/MAGS welding machine                                                                  (1)

(7)

| 6.1.2 | A. | Welding pistol/gun  |
| --- | --- | --- |
|  | B. | Switch  |
|  | C. | Regulator or Gas flow meter  |
|  | D. | Gas cylinder  |
|  | E. | MIGS/MAGS welding machine  |
|  | F. | Earth cable  |
|  | G. | Welding pistol conduit/welding hose  |

6.2         Operating principles of an X-ray testing equipment:

•     The X-ray source is placed in front of the object being tested. 

•	The source is activated for a brief moment and the X-rays penetrate the test piece. 

•	As the X-rays pass through areas of lower density, it will be exposed lighter on the film, which indicates the welding defects. 

•     A photographic film with details of defects is provided, which can be

studied.                                                                                                                             (6)

6.3         Advantages of metal-arc shielded welding (MIGS/MAGS):

•     Can weld in any position 

•     Less operator skills are required 

•     Long welds can be made without stops and starts 

•     Minimal post welding cleaning is required 

(Any 3x1)         (3)

6.4         Bend test:

•      To measure ductility of the weld deposit and the heat affected area

adjacent to the weld.                                                                                            (2)

6.5         Welding defects:

6.5.1       Incomplete penetration:

•     Welding speed too high 

•     Joint design faulty 

•     Arc is to long 

•     Current too low 

(Any 2x1)         (2)

6.5.2       Welding craters:

•     Current too high 

•     Incorrect welding technique 

•     Electrode too thin 

(Any 2x1)         (2)

6.6         Welding techniques:

•    Rate of electrode burning and progress of the weld 

•    The angle of the electrode 

•	The distance between the parent metal and the electrode. (Arc length) 

(Any 2x1)         (2)

[25]

QUESTION 7: FORCES

7.1         Equilibrant:

210sin25°

210 N



210cos25°

250sin45°

250cos45°           

250 N

25°           45°

360 N

150 N

∑ HC = 360 + 250cos45° − 210cos25°                    

= 346,45 N                                                    

∑ VC = 250sin45° + 210sin25° − 150                     

= 115,53 N                                                     

OR

| HORIZONTAL COMPONENT | MAGNITUDES | VERTICAL COMPONENT | MAGNITUDES |
| --- | --- | --- | --- |
| -210 cos25 0  | -190,32N | 210 sin25 0  | 88,75 N |
| 250 cos45 0  | 176,78N | 250 sin45 0  | 176,78 N |
| 360 | 360 N | -150 | -150 N |
| TOTAL | 346,45 N  | TOTAL | 115,53 N |

HC = 346,45 N  



VC = 115,53 N

E                                                

E2 = HC2 + VC2                                                                    

E  =   346,452 + 115,532                                                 



E  = 365,21 N

θ

Tan

=  VC



HC                                                         

=  115,53

346,45

θ

=18,440                                                                            

E   = 365,21 N at 18,440 south from west



(15)

7.2         Stress and Strain:

Stress:

A = L2                                        

A = 0,12

A = 0,01 m2                           

σ

=  F                              

A

80 × 103

σ

=

0,01                      

σ   σ

= 8 × 106 Pa



= 8 MPa

(5)

7.3         Strain is directly proportional  to the stress  that causes it, provided

the limit of elasticity is not exceeded.                                                                    (3)

7.4         Moments:

50 N/m

UDL

400 N        600 N

1,5 m                5 m

2 m                 4,5 m

A                                                                                       B

Calculate A

Take moments about 'B'

A × 11,5 = (600 × 4,5) + (400 × 6,5) + (325 × 9,75)                 

= 2700 + 2600 + 3168,75

= 8468,75  N                                                                

A =  8468,75

11,5

A = 736,41  N                                                                 

Calculate B

Take moments about “A”

B × 11,5 = (325 × 1,75) + (400 × 5)+ (600 ×7 )                               

= 568,75 + 2000 + 4200

= 6768,75  N                                                                 

B =  6768,75                                                                      

11,5

B = 588,59  N

(7)

[30]

QUESTION 8:  MAINTENANCE

8.1          Advantages of cutting fluid:

•     The workpiece and cutting tool are kept cool. 

•     The life of the cutting tool is prolonged. 

•     A better finish is imparted to the workpiece. 

•     Cuttings are washed away. 

•     The worker is protected from very fine metal chips and dust. 

•     It prevents corrosion. 

•     Productivity is increased because the cutting process is faster. 

|  | (Any 2x1) | (2) |
| --- | --- | --- |
| 8.2 | Preventive maintenance is maintenance of equipment and systems before faults occur. | (1) |
| 8.3 | Chain drive: |  |

8.3.1         Chain drive preferable to belt drive:

•     It is much stronger 

•     It has a much longer service life 

•     It provides positive drive. (No slip) 

(Any 2x1)          (2)

8.3.2          Stretched chain:

•     The chain loses its strength/tension. 

•     It generates more friction. 

•     It causes the chain to vibrate. 

•     It causes excessive noise. 

•     The chain can break. 

•     The chain can easily slip from its sprocket. 

(Any 2x1)          (2)

8.3.2          Chain Replacement:

| • Disconnect the link plate.  |  |
| --- | --- |
| • Remove the chain from the sprockets.  |  |
| • Select the correct length and size replacement chain.  |  |
| • Fit the new chain.  • Insert the link plate and tension the chain.  | (6) |

•	Align crankshaft and camshaft pulleys before removing the timing chain.

8.4         Engine oil must have a high flash point to prevent the vapour to ignite.                (2)

[15]

QUESTION 9: SYSTEM AND CONTROLS

9.1         Gear drives:

9.1.1         Number of teeth on idler gear:

TB × NB  = NA  × TA                                              

= T   × N

T

A           A

B

NB



T  =  50 × 660

B           1000

TB  = 33 teeth                                

(3)

9.1.2         Rotation frequency of the driven gear:

NC ×TC = NA × TA                        

C

N   = NA  × TA

TC



N   =  660 × 50

C               60

N   =  550 rpm

C               60

NC  = 9,17 r/s                    

(3)

9.2         Belt Drives:

9.2.1         Rotation frequency of the driven pulley:

NDN × (DDN  + t) = NDR × (DDR  + t)   

=  NDR × (DDR  + t )

NDN

(DDN

+ t)

NDN  =

1640 × (175 + 12)             

(80 + 12)

NDN

=  3333,48 rpm

60



NDN  = 55,56 r/s

(3)

9.2.2         Belt speed:

π

v =

π

v =

(D + t)N

60                                 

(0,175 + 0,012)× 1640     

60

v = 16,06 m/s                            

(3)

9.3         Hydraulics:

9.3.1         Fluid pressure:

π

2

π

A A  = D

4

(0,038)2

A A  =

4

A

A   = 1,13 ×10−3                             

p =  FA                                                      

A A

p =      240

1,13 ×10−3

p = 211618,76 Pa              

9.3.2         Force exerted by piston B:

πD 2                                            

(3)

AB  =

AB =

4

π (0,15 )2

4

AB = 0,017671458 m2                         

B

F P =

AB                                              

FB = P × A

FB = (211618,76 )× (0,017671458 )

FB = 3739,61  N                                  

(4)

9.4         Purpose of vehicle engine management system:

The vehicle engine management system controls the...

•     Engine fuel system 

•     Ignition system 

•     Exhaust emission 

•     Cooling system 

•     Battery charging system 

(Any 4x1)          (4)

| 9.5 | Purpose of anti-lock brake system: |  |
| --- | --- | --- |
|  | ABS relieves hydraulic pressure on wheels which are about to skid.  action reduces the braking effort that would have caused a skid.  OR | This |

The  purpose  is  to  provide  safer  vehicle  handling     under  difficult

conditions.                                                                                                                            (2)

[25]

QUESTION 10: TURBINES

10.1       Water turbine:

•     Water turbines do not emit carbon. 

•     No water is destroyed in the process of creating electricity. 

•     Water turbines are more reliable. 

•	Water turbines continue to turn on cloudy and windless days unlike solar and wind operated generators. 

•      Environmental friendly and no pollution. 

(Any 2x1)          (2)

| 10.2 | Water turbine definitions: 10.2.1 Specific speed of a water turbine is the speed at which the |  |
| --- | --- | --- |
|  | turbine turns for a particular discharge with the unit head, and thereby is able to produce unit power.  | (2) |
|  | 10.2.2 Free load/runaway speed of a water turbine is its speed at full flow and with no shaft load.  | (2) |
| 10.3 | A steam turbine is a mechanical device that extracts thermal energy from pressurised steam and converts it into useful mechanical work. | (2) |
| 10.4 | Classification of steam turbine: |  |
|  | • Condensing turbines  |  |
|  | • Non-condensing turbines  |  |
|  | • Reheat turbines  |  |
|  | • Extraction turbines  |  |
|  | • Induction turbines  (Any 3x1) | (3) |
| 10.5 | Gas turbine for naval vessels: It is valued for their high power to weight ratio which has quick acceleration as result.  | (2) |
| 10.6 | Turbo boost: Turbo boost refers to the increase in manifold pressure that is generated by turbocharger in the intake path or specifically intake manifold that exceeds atmospheric pressure | (2) |

10.7       Operation of twin-screw supercharger:

•	A twin screw supercharger operates by pulling air through a pair of meshing lobes that resemble set of worm gears.

•	The air inside a twin screw supercharger is trapped in pockets created by the rotor blades.

•     A twin screw supercharger compresses the air inside rotor housing.

•	Reason is the rotors have a conical taper which means the air pockets decrease in size as air moves from the fill side to the discharge side.

•     As the air pockets shrink, the air is squeezed into a smaller space and

increases the pressure.                                                                                            (5)

[20]

GRAND TOTAL:          200